IEEE P802.21/D01.09
Draft IEEE Standard for Local and Metropolitan Area Networks:
Media Independent Handover Services

| Normative Text Proposal for QoS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-September-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Nada Golmie | National Institute of Standards and Technology | 100 Bureau Dr, Stop 8920 Gaithersburg, MD | +1301-975-4190 | Nada.golmie@nist.gov |
| Ulises Olvera | InterDigital Communications | 100, rue Sherebrook, Ouest, 10e étage, Montreal, QC, H3A 3G4 | +1514-904-6258 | Ulises.Olvera-Hernandez@InterDigital.com |
| Reijo Salminen | Seesta | Kumitehtaankatu 5, FIN-04260 KERAVA, FINLAND | +358-9-68995288 | Reijo.Salminen@seesta.com |
| Cristian Lambiri | IDT Canada | 450 March Road, Kanata, Ontario, K2K 3K2 Canada | + 1 613 287 5133 | clambiri@idt.com |

- Abstract

This document contains a normative text proposal in support of a Media Independent Handover Quality of Service model.

Notice: This document has been prepared to assist IEEE 802.21. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Proposed MIH QoS Model and normative text modifications to the IEEE 802.21 standard draft D01.09.

We propose to add to Section 3 on page 9 the following definitions:

Population of interest: the set of packets corresponding to a given source and destination over which the QoS parameters are computed.

We propose to replace section 5.1.3 on page 13 with the following text specified in blue:

5.1.3 Packet Transfer Quality Model

The quality of the service (QoS) experienced by an application depends on the accuracy, speed  and dependability of information transfer of the communication channel.  The IEEE 802.21 specifications provide support for fulfilling application QOS requirements in the presence of communication handover. The MIH QoS model defines parameters that may be used in specifying and assessing characteristics of packet transfers between a source and its destinations. The communication channel is considered to be composed of several connected segments, each under a possibly different but cooperative administrative authority. Examples for such channels, such as for IP traffic have been detailed in ITU normative Y.1540.

In the context of the 802.21 specification it is assumed that applications communicate via a packetized communication channel over which a transmitted packet can experience the following outcomes:

- Be received with no errors at its intended destination

- Be received with errors at its intended destination

- Not be received in which case it is said that the packet is lost.

The maximum attainable speed of information transfer over a given communication channel can be constant, as it is usually the case with network segments involving only wired links, or it can be time varying at different scales, at is the case for segments involving wireless links. This measure will be called channel capacity, for the purposes of this document.

The ability of the channel to provide accurate information transfer can be described via a statistical model characterized by the following parameters:

- Minimum Packet Transfer Delay: is defined as the minimum delay over a population of interest

- Average Packet Transfer Delay: is defined as the arithmetic mean of the delay over a population of interest

- Maximum Packet Transfer Delay: is defined as the maximum delay over a population of interest

- Jitter: is defined as the standard deviation of the delay over a population of interest

- Packet loss rate: is defined as the ratio between the number of frames that are transmitted but not received and the total number of frames transmitted over a population of interest.

- Packer error rate: is defined as the ratio between the number of packets that have been received with errors and the total number of packets present in a population of interest.

It is generally accepted that based on the required accuracy of information transfer applications can be grouped into a small number of behavioral sets [Y.1541] called Classes of Service (CoS). Support for differentiation via Classes of Service is pervasive in many of the IEEE 802 based standards [802.11, 802.1q, 802.16 etc.].

For a link that supports CoS differentiation, per CoS traffic accuracy parameters need to be maintained in order to provide insights on how individual traffic classes are faring.

In summary, the following set of parameters characterize the speed and accuracy of the information transfer a multi-CoS traffic link supports:

- Link capacity, representing the maximum information transfer rate achievable. This value is determined by the physical characteristics of the link. While for wired links is it usually a constant, for wireless links it is time varying.

- Link Packet Error Rate: representing the ratio between the number of frames received in error and the total number of frames transmitted in a link  population of interest

- Supported classes of service: represents the maximum number of differentiable classes of service supported by this link.

- Class of Service Parameters List. For each of the supported classes of service the following parameters are defined:

  - Class Minimum Packet Transfer Delay: is defined as the minimum delay over a class population of interest

  - Class Average Packet Transfer Delay: is defined as the arithmetic mean of the delay over a class population of interest

  - Class Maximum Packet Transfer Delay: is defined as the maximum delay over a class  population of interest

  - Class Packet Delay Jitter: is defiend as the standard deviation of the delay over a class population of interest

  - Class Packet loss rate: is defined as the ratio between the number of frames that are transmitted but not received and the total number of frames transmitted over a class population of interest.

The performance implications to consider from the MIH perspective include both the transient network performance achieved during a handover as well as the continuous monitoring of current network conditions.

The 802.21 specifications provide mechanisms to support seamless mobility as an enabler; however the MIHF alone cannot guarantee seamless mobility. Depending on the QOS requirements of the end-to-end application, seamless mobility implies minimizing the latency, and potential loss incurred during a handover so as to minimize the end-to-end delay and loss perceived by the application. Seamless mobility also implies the timely assessment of network conditions, such as the monitoring of the signal strength, packet loss, on the link, on both current and target networks in order to optimize the handover decision and its execution.

The MIH services defined by the IEEE 802.21, including event, command, and information service, need to consider network traffic performance objectives and their potential fulfilment as well as their relation to  application traffic quality of service requirements.

In order to realize the QoS model described above, a number of primitives require modification to conform to the model. Modifications are applicable to all primitives that include the LINK_QoS_PARAMETER_LIST information element. Within the current draft, the LINK_Qos_PARAMETERS_LIST IE should be contained in the following messages:

  - MIH_Configure.request

  - MIH Get Status.confirm

  - MIH_Link_Parameter_Report.indication

Therefore, the following section should be updated as follows:

Section 7.4.8.1.2  Semantics of Service Primitive

The primitive parameters are as follows:

MIH_Configure.request (

SourceIdentifier

DestinationIdentfier

LinkIdentifier

ConfigurationRequestSets

)

Table 19 details the set of parameters that can be configured:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| OPERATION_MODE | INTEGER | 0x01 – Normal Mode 0x02 – Power Saving Mode 0x03 – Power Down | Change the device’s power mode |
| DISABLE_TRANSMITTER | Boolean | N/A | Enable/disable the transmitter of the interface |
| LINK_ID | Link ID | N/A | Change to the specified link |
| CURRENT_ADDRESS | STRING | N/A | Change the current address to the value specified |
| SUSPEND_DRIVER | Boolean | N/A | Suspend or resume of the specified interface |
| LINK_QoS_PARAMETERS_LIST | LIST | N/A | A list of QoS Parameters and their corresponding values as specified in table XX |

Insert the following table in section 7.4.8.1, after line 53

Table XX: LinkQoSParameterType values

| Value | Name | Value Size (octets) | Valid Range | Description |
| --- | --- | --- | --- | --- |
| 0 | Throughput | 4 | 0-(232-1) | The maximum information transfer rate achievable. This value is determined by the physical characteristics of the link . It is measured in kbps |
| 1 | Link Packet Error Rate | 2 | 0-65535 | A value equal to integer part of the result of multiplying -100 times the log10 of the ratio between the number of packets received in error and the total number of packets transmitted in a link population of interest. |
| 2 | Supported number of COS | 2 | 0-65535 | The maximum number of differentiable classes of service supported by this link |
| 3 | CoS Minimum Packet Transfer Delay | 4 | 0-2^32 | This is an encoded value which contains the class of service identifier in the 2 most significant octets and the minimum packet transfer delay for the class in ms in the two least significant octets. Valid range for minimum packet transfer delay: [0..65535] ms |
| 4 | CoS Average Packet Transfer Delay | 4 | 0-2^32 | This is an encoded value which contains the class of service identifier in the 2 most significant octets and the average packet transfer delay for the class in ms in the two least significant octets. Valid range for average packet transfer delay: [0..65535] ms |
| 5 | CoS Maximum Packet Transfer Delay | 4 | 0-2^32 | This is an encoded value which contains the class of service identifier in the 2 most significant octets and the maximum packet transfer delay for the class in ms in the two least significant octets. Valid range for maximum packet transfer delay: [0..65535] ms |
| 6 | CoS Packet Transfer Delay Jitter | 4 | 0-2^32 | This is an encoded value which contains the class of service identifier in the 2 most significant octets and the packet transfer delay jitter for the class in ms in the two least significant octets. Valid range for packet transfer delay jitter: [0..65535] ms |
| 7 | CoS Packet Loss rate | 4 | 0-2^32 | This is an encoded value which contains the class of service identifier in the 2 most significant octets and a value equal to integer part of the result of multiplying -100 times the log10 of the ratio between the number of packets lost and the total number of packets transmitted in the class population of interest |
| 8-255 | Reserved | N/A | N/A | Reserved for future use. |

With regards to primitive “MIH_Get_Status.confirm”, made the following modification to the table contained in section 7.4.6.2.4 “Effect of receipt”

The following table details the set of parameters for which status can be provided

| Name (Get Status Requests) | Type | Valid Range | Description |
| --- | --- | --- | --- |
| DEVICE_INFO | STRING | N/A | Information on manufacturer, model number, revision number of the software/firmware and serial num ber in displayable text are returned. |
| OPERATION_MODE | INTEGER | 0x00 Normal Mode 0x01 Power Saving Mode 0x02 Power Down | Returns the link’s current power mode. |
| LINK_ID | Link ID | N/A | Return the ID of the link currently configured to communicate with. |
| CHANNEL_ID | INTEGER | N/A | The ID of the channel currently in use |
| CHANNEL_QUALITY | STRING | N/A | Current Connection Quality |
| LINK_SPEED |  | N/A | Bits per second |
| BATTERY_LEVEL | INTEGER | 0-100; -1; unknown bat tery level | Battery level in percentage; |
| LINK_QoS_PARAMETER_LIST | LIST | N/A | A list of QoS Parameters and their corresponding values as specified in table XX |

With regards to primitive “MIH_Link_Parameters_Report.indication”, made the following modification to the table contained in section 7.4.5.1.1 “Function”

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| LINK_IDENTIFIER | Link ID | N/A | Identifier of the link associated with the event |
| NumLinkParameter | INTEGER | 0-65535 | Number of entries in LinkParameterList |
| LINK_Qos_PARAMETER_LIST | List | N/A | A list of QoS Parameters and their corresponding values as specified in table XX. |

In page 157, add after line 60 the following table:

Annex A-5: QoS

Delay_Var_Sensi

tive

Tolerated Jitter

Jitter

Max_Latency

Transfer Delay

Maximum Latency

Delay Bound

Delay

Packet_Size

Maximum SDU

Size

Maximum MSDU

Size

Max Packet Size

SDU Error Ratio

Packet Error Rate

Packet Error Rate

Max_IP_Packet_

Loss_Rate

Packet Loss Rate

Before Retransm.

Peak Data Rate

Guaranteed

Bitrate

Minimum Reserved

Traffic Rate

Minimum Data Rate

Guaranteed (Min)

Bitrate

Maximum

Bitrate

Maximum Sustained

Traffic Rate

Max Bitrate

3GPP2

3GPP

802.16

802.11

802.21

Delay_Var_Sensi

tive

Tolerated Jitter

Jitter

Max_Latency

Transfer Delay

Maximum Latency

Delay Bound

Delay

Packet_Size

Maximum SDU

Size

Maximum MSDU

Size

Max Packet Size

SDU Error Ratio

Packet Error Rate

Packet Error Rate

Max_IP_Packet_

Loss_Rate

Packet Loss Rate

Before Retransm.

Peak_Rate

Guaranteed

Bitrate

Minimum Reserved

Traffic Rate

Minimum Data Rate

Guaranteed (Min)

Bitrate

Maximum

Bitrate

Maximum Sustained

Traffic Rate

Max Bitrate

3GPP2

3GPP

802.16

802.11

802.21

Table yy: An example of a QoS Parameter Mapping Table

Figure 1: Event Flow Diagram: Initial Stage

Figure 2: Event Flow Diagram: Link Selection

Figure 3: Event Flow Diagram: Handover