# CIS2 Recommended Practices and Issues

Robert Lipman, NIST, 8/5/02

1.  Element orientations for ELEMENT_CURVE_SIMPLE are specified by DIRECTION are done incorrectly.  The orientation is the rotation of the element about the longitudinal axis, not the slope of the element.  The orientation can be specified by a beta angle or DIRECTION.  The values for DIRECTION can be computed directly from the beta angle.

The documentation for element orientation (p128-132) needs to clarified.  There should be a picture of an example structure with several beams, columns, sloped elements, with and without an element orientation with their beta angles and DIRECTION specified.

Many of the vendor supplied and the Georgia Tech element orientation sample CIS2 files are incorrect.

2.  Section profiles can be specified with either SECTION_PROFILE or with the more complete form SECTION_PROFILE_I_TYPE, etc.  When using S_P_I_TYPE the actual section dimensions are specified.  Using S_P_I_TYPE eliminates any uncertainty in the dimensions but requires more work to import and export.  S_P_I_TYPE is useful for nonstandard section profiles.

Most sample CIS2 files use only S_P.  SDS/2 CIS2 files use S_P_I_TYPE.  S_P_I_TYPE has been used incorrectly without the section dimensions, which are not optional.

3.  Although the coordinate system associated with a LOCATED_JOINT_SYSTEM should be at the “end” of the JOINT_SYSTEM_MECHANICAL for bolts, this is not being done in practice.  Both SDS/2 and Xsteel are using the underside of the bolt head as the origin of the coordinate system.  The VRML translator also makes this assumption.  No other fastener types are being used.  For other fasteners, such as shear studs, the origin would probably be at the “end” of the shear stud.

Having the origin at the underside of the bolt makes it easier to place the holes relative to the bolts.  If the origin of the coordinate system for the holes, which are located relative to the part, is on the surface of the part, then it is a simple transform to get the origin of the coordinate system for the bolts that are located relative to the assembly.

The idea that the origin for the bolts should be at the top of the head came from a CIS/1 diagram.

4.  No one is using FASTENER_SIMPLE_BOLT_HEXAGONAL_HEAD or any other type of head which would specify the bolt head height.  There is no way to specify the location of the nuts and washers relative to the bolt.

5.  In theory the origin of the coordinate system for locating holes on a part need not be on the surface of the part.  By default holes are drilled through all material along the x-axis of the coordinate system.  The depth of the hole can be limited by FEATURE_VOLUME_CURVE.  The depth is the thickness if the origin is on the surface.  It should be recommended that the origin for holes lie on the surface of the part.

There might be an easier way to specify if the hole goes through one surface (flange) or all surfaces (both flanges) so that the thickness of the flange does not have to be specified with FEATURE_VOLUME_CURVE.

6.  The numeric precision of values should be considered.  Do coordinates specified in millimeters really need 16 digits of precision?  Will truncating values cause problems in the software that is importing a file?  Displacements should be output as 1.2x10-6 rather than 0.0000012.  Directions that should be aligned exactly with the x, y, or z axes should be checked.

7.  Some CIS2 files have attempted to do a skewed end cut with FEATURE_CUTTING_PLANE rather than FEATURE_VOLUME_PRISMATIC_SKEW_END.  Although it’s valid to use a cutting plane, it’s easier to specify the angles for the skewed end cut.  When is it valid to use a cutting plane?

8.  Features such as notches, chamfers, and skewed ends are located by top, bottom, left, right, start, end and a coordinate system.  The coordinate system is redundant information and might be difficult to compute.  Should there be a FEATURE_FOR_LOCATED_PART so that the features do not also have to be located with a coordinate system as with LOCATED_FEATURE_FOR_LOCATED_PART?

9.  If the bottom of the top chord of a joists rests on top of a beam, there is no cardinal point that can be use to specify this.  There needs to be an arbitrarily positioned cardinal point.  The VRML translator assumes that the cardinal point specified is relative to the top chord and not the entire depth of the chord.  This results in joists that are positioned correctly.  This is valid only if the truss is attached to the beam with the top chord.

10. Other than the throat thickness no other weld dimensions or cross section can be specified.

11.  There is no way to specify the designator for plates, i.e. PL10x3/8, as there is with prismatic members where the designator appears in SECTION_PROFILE.  SDS/2 is using STRUCTURAL_FRAME_ITEM(STRUCTURAL_FRAME_PRODUCT)

12.  A twisted prismatic member can only be specified with PART_PRISMATIC_COMPLEX that refers to multiple cross sections and orientations.  A twisted member would have a constant cross section and twist angle.  Should there be a PART_PRISMATIC_SIMPLE_TWIST that specifies a twist angle?

13. The CURVE in PART_PRISMATIC_SIMPLE_CURVED should only be used to specify truly curved members.  If it is used for straight members, then the CURVE should be a LINE whose CARTESIAN_POINT (origin) is 0,0,0 and VECTOR has a magnitude of 1. and DIRECTION of 1,0,0.  The CURVE should not be used to locate the part.  Straight parts should use PART_PRISMATIC_SIMPLE.

14.  Bent plates can be modeled with SECTION_PROFILE_CENTERLINE.  The centerline is the bend and “length”.  The length of the part is the width of the bent plate.  SDS/2 is doing it this way.

15.  Grates can be modeled as plates or individual bars.  Design Data has done a lot of work to model grates.  If grates are modeled as individual bars, then the bar parts refer to a LOCATED_ASSEMBLY that in turn refers to another LOCATED_ASSEMBLY.  This way all the individual grating bars are grouped together in its own assembly.

16.  There should be an agreement as to how ladder rungs are modeled so that they are transferred correctly.  Ladder rungs can be modeled as plates (SDS/2) or prismatic parts (Bentley).

17.  Several other problems with CIS2 files are detected by the VRML translator that other STEP tools do not.

Element orientation units in radians but the angle (90 or –90) is obviously in degrees.

Negative values in POSITIVE_LENGTH_MEASURE

Depth, width, or length of features, such as notches or chamfers, that are zero.

Fastener mechanisms that do not reference a bolt.

Missing mirror field in SECTION_PROFILE.

Cardinal point values greater than 9.  Will those values be properly interpreted and used.

Truncated section profile designator, L3-1/2x3-1/2x1/16  The string is 17 characters long but only 16 were output.

Section profile in wrong field.