Proceedings

of the

Workshop on Future Large

CO2 Compression Systems

March 30-31, 2009

National Institute of Standards and Technology

Gaithersburg, MD

Sponsored by

DOE Office of Clean Energy Systems

National Institute of Standards and Technology

EPRI

## Prepared By

Ronald H. Wolk

Wolk Integrated Technical Services

San Jose, CA

July 30, 2009

DISCLAIMER OF WARRANTIES AND

LIMITATIONS OF LIABILITIES

This report was prepared by Wolk Integrated Technical Services (WITS) as an account of work sponsored by Pacific Northwest Laboratories

WITS: a) makes no warranty or representation whatsoever, express or implied, with respect to the use of any information disclosed in this report or that such use does not infringe or interfere with privately owned rights including any party's intellectual property and b) assumes no responsibility for any damages or other liability whatsoever from your selection or use of this report or any information disclosed in this report.

Table of Contents

| Section | Title | Page |
| --- | --- | --- |
| 1 | Summary | 1 |
| 2 | Overview of Technical Presentations A. Sources of CO2 in the US B. CO2 Capture Technology C. CO2 Pipelines D. Delivered Cost of CO2 E. Challenges of CO2 Transportation F. Properties of CO2 and Co-constituents Near the CO2 Critical Point G. Compression Systems Machinery H. Electric Drive Machinery I. Drive Electronics and Components | 4 5 7 8 8 9 11 14 16 |
| 3 | Prioritization of Potential R&D Projects | 19 |
| 4 | List of Workshop Presentations | 27 |
| 5 | Appendices |  |
|  | - Workshop Agenda | 28 |
|  | B. List of Workshop Participants | 31 |

List of Abbreviations

3D		three dimensional

A		amperes

AC		Alternating Current

acfm		actual cubic feet per minute

API		American Petroleum Institute

Bar		metric unit of pressure, approximately 14.5 psi

Bara		bar, absolute

bcf		billion cubic feet

C		Centigrade

CCS		Carbon Capture and Sequestration

CTE		Coefficient of Thermal Expansion

ERDC-CERL 	US Army Engineer Research and Development Center, Construction
      Engineering Research Lab

EPRI	Electric Power Research Institute

d		day

DC		Direct Current

DMOSFET     Double Diffused (or Implanted) Metal-Oxide-Semiconductor Field Effect
      Transistor

DOD		Department of Defense

DOE		Department of Energy

EOR

EOS		Equation of State

F		Fahrenheit

FC 		Fuel Cell

GW		Gigawatt

Gt		Giga-tonnes

GTO		Gate Turn-Off Thyristor

HANS             HANS equation of state

HF		High Frequency

Hz 		Hertz

hr		hour

HVDC		High Voltage Direct Current

HV		High Voltage

IEA		International Energy Agency

IGBT		Insulated Gate Bipolar Transistor

IGCT		Integrated Gate Commutated Thyristor

kA		kilo-amperes

kHz                 kilohertz

km		kilometer

kV		kilovolt

kVA		kilovolt ampere

kW		kilowatt

kWh		kilowatt hour

lbm/hr 		pound moles/hour

LCI 		Line Commutated Inverter

LMTD		Log Mean Temperature Difference

LNG		Liquefied Natural Gas

MEA		Monoethanolamine

MERGE	Model for Evaluating the Regional and Global Effects of GHG Reduction
      Policies

M/G		Motor/Generator

MM 		million

MMSCFD	million standard cubic feet per day

MSCF		thousand standard cubic feet

MOSFET	Metal-Oxide-Semiconductor Field Effect Transistor

mt		metric tonnes

mt/yr		metric tonnes per year

MVA		Megavolt Ampere

MW		Megawatt electric

MWt		Megawatt thermal

NIST  		National Institute of Standards and Technology

Nm3		Normal cubic meters

PCS		Power Conditioning System

psia		pounds per square inch absolute

PVT		Pressure Volume Temperature

ppm		parts per million

R&D		Research and Development

RKS		Redlich-Kwong-Soave equation of state

rpm		revolutions per minute

SwRI		Southwest Research Institute

tpd		tons/day

V		volts

VLE		Vapor Liquid Equilibria

- Summary

A Workshop on Future Large CO2 Compression Systems was held on March 30-31, 2009 at NIST headquarters in Gaithersburg, MD. Such systems could be utilized as part of the equipment needed to transport CO2 captured at fossil fuel power plants by pipeline to permanent sequestration sites and/or for sequestration well injection. Seventy-seven people who are active in this field participated. The Organizing Committee for the Workshop consisted of Dr. Allen Hefner of NIST, Dr. Robert Steele of EPRI, Dr. Peter Rozelle of DOE and Ronald H. Wolk of Wolk Integrated Technical Services.

The objective of this Workshop was to identify and prioritize R&D projects that could support development of more efficient and lower cost CO2 compression systems. Reducing the total cost of Carbon Capture and Sequestration is a major goal of R&D programs sponsored by organizations including US DOE, IEA, EPRI, MERGE and others. The capital cost of compression equipment and the associated cost for compression energy are major components of this total cost.

Twenty technical presentations were given to familiarize Workshop participants with a broad spectrum of multiple aspects of the technologies involved including:

- Future Market Drivers for CO2 Compression Equipment

- Characteristics of Large Power Plants Equipped for CO2 Capture and Compression

- Oil and Gas Industry Experience with CO2 Capture, Compressors and Pipelines

- Compressor Vendor Perspective on Changes in Compression Cycle Machinery

- Electric Drive Compressor Potential for Improvement in Capitol Cost, Power Requirements, Availability, and Safety

- Advanced Compressor Machinery Future R&D Needs

- Advanced Electric Drive Compressor Future R&D Needs

The presentations are available at www.nist.gov/eeel/high_megawatt/2009_workshop.cfm

The key points that can be summarized from these presentations are that:

- Existing commercial CO2 pipelines in the United States, with a total length of about 5650 km (3500 miles), operate safely

- These pipelines are utilized primarily to deliver about 68,000 mt/day (75,000 tons/day) of pressurized CO2, recovered from both natural reservoirs and from natural gas purification and chemical plants to existing Enhanced Oil Recovery projects.

- A typical 550 MW coal-fired power plant will produce about 13,500 mt/day (15,000 tons/day) of CO2.  A large number of coal-fired power plants of this size are likely to be built between now and 2030 to meet the increased demand for power in the US. According to the EIA AEO2009 reference case, total electricity generation from coal-fired power plants will increase from 1906 billion kWh in 2009 to 2236 billion kWh in 2030. The current capacity of coal fired generating plants in the US is about 311,000 MW.

- The accuracy of the Equations of State used to predict the properties of the CO2 recovered from the flue gas  produced by coal-fired power plants, which includes a wide variety of contaminants, needs to be improved to reduce typical design margins used by compressor vendors.

- Reciprocating and centrifugal compressors are available from a variety of vendors to meet the pressure and volumetric flow requirements of all applications. The largest machines pressurize about 18,000 mt/day (20,000 tons/day) to 27,000 mt/day (30,000 tons/day) of CO2 to the pressures required for pipeline transportation or sequestration well injection.

- Power required for compression could be reduced if CO2 was first compressed to an intermediate pressure, then cooled and liquefied, and that liquid is then pumped to the higher pressure level required for pipeline injection.

- Improved materials are needed to allow higher speed rotor operation and corrosion resistance of rotors and stators.

- Competitively priced commercially available power conditioning components and modules are needed that will allow systems to operate at >10 kV and switch at >10 kHz

- SiC-based power conditioning and control components to replace existing Si-based components can lead to higher efficiency electric drive systems.

After digesting the information presented, the Workshop participants suggested a total of 33 R&D projects in seven categories. Thirty-seven of the Workshop attendees then participated in a Prioritization Exercise that allocated 3700 votes (100 by each of those participants) among the seven categories of R&D activities and 33 specific R&D projects.

The results of the Prioritization Exercise are presented in Tables 1 and 2. Table 1 lists the rank order by total votes of the seven Categories. Table 2 lists the top 10 projects, out of a total of 33, by rank order of total votes.

Table 1.  Rank Order of R&D Categories

| R&D Categories | Total Votes |
| --- | --- |
| 1. Properties of CO2 and Co-constituents | 914 |
| 2. Integration of CO2 Capture and Compression | 726 |
| 3. Compression Systems Machinery and Components | 690 |
| 4. Electric Drive Machinery | 545 |
| 5. Pipeline Issues | 456 |
| 6. Drive Electronics and Components | 326 |
| 7. Impacts of Legislation on CCS | 43 |

Table 2. Rank Order of Top 10 R&D Projects

| R&D Project | Total Votes |
| --- | --- |
| 1. Perform more gas properties measurements of CO2 mixtures | 435 |
| 2. Improve Equations of State | 401 |
| 3. Optimize integration of a CO2 capture/compression system together with the power plant | 280 |
| 4. Comparison and evaluation of compression-liquefaction and pumping options and configurations | 204 |
| 5. Higher voltage, higher power, and speed electric motors and drives | 165 |
| 6. Install test coupons in existing CO2 pipelines to obtain corrosion data, then develop CO2 product specifications | 150 |
| 7. Determine optimal electric motor and drive types, speeds, and needed voltages, etc., for CO2 compressors | 143 |
| 8. Establish allowable levels of contaminants in CO2 pipelines and/or compressors | 120 |
| 9. Compressor heat exchanger data for power plant applications including supercritical fluids | 117 |
| 10. Integrate utilization of waste heat to improve cycle efficiency | 113 |

- Overview of Technical Presentations

This section of the report organizes a fraction of the total information presented at the Workshop into brief summaries. Readers are strongly encouraged to review the actual presentation materials for those topics about which they need additional information.

A.   Sources of CO2 in the US

CO2 is recovered commercially from a variety of sources including natural sealed reservoirs typically referred to as domes, and industrial plants. High purity (>95%) CO2 gas streams are available from processing plants that purify raw natural gas to meet standards for pipeline transmission, and from chemical plants that gasify coal or produce hydrogen, ammonia, and other fertilizers, and potentially from future gasified coal power plants. These operations are the preferred man-made sources of CO2 because the gas from those plants is available at high pressure. Other sources of CO2 are available at lower pressures at high purity (from fermentation plants producing ethanol) and at low purity (from pulverized coal power plants and cement plants). The locations of various commercially utilized sources of CO2 are listed below and are also shown in Figure 2.1 (Kubek)

- Natural CO2 Reservoirs

  - Bravo Dome (TX)

  - Jackson Dome (MS)

  - McElmo Dome (CO)

  - Sheep Mountain Dome (CO)

- Natural Gas Purification Plants

  - LaBarge Gas Plant (WY)

  - Mitchell Gas Plant (TX)

  - Puckett Gas Plant (TX)

  - Terrell Gas Plant (TX)

- Solid Fuel Gasification Plant

  - Great Plains Coal Gasification Plant (ND) – fueled with North Dakota lignite (2.7 million tons CO2 per year)

  - Coffeeville Resources Plant (KS) – fueled with Coffeeville refinery petroleum coke

- Industrial Chemical Plants

  - Ammonia Plant (OK)

Low purity CO2 containing streams are produced by coal-fired power plants (12-15%), cement plants (12-15%), and natural gas fired gas turbine/combined cycle power plants (3-4%). These are not used as sources for large scale CO2 recovery. (Schoff)

Much of the CO2 that is separated in natural gas purification systems is not utilized commercially but is disposed of by venting to the atmosphere, or if contaminated with H2S, is injected into saline aquifers through deep injection wells. Over 50 acid gas (CO2 + H2S) injection projects for acid gas disposal are currently operating in North America. In most cases the acid gases consist primarily of H2S but all streams contain CO2. Injection rates range from    < 0.0268 MM Nm3 (<1 MMSCFD) to 0.48 MM Nm3 (18 MMSCFD) in Canada. The ExxonMobil LaBarge Gas Plant in Wyoming injects about 2.4 MM Nm3 (90 MMSCFD). Major process components after the Acid Gas Removal plant are either compression with integrated partial dehydration or compression and standard dehydration. Various conceptual projects are in the design stages in the Middle East for acid gas injection rates that will exceed 10.7 MM Nm3  /day (400 MMSCFD). (Maddocks)

Existing acid gas injection plants typically use reciprocating compressors. Larger volume conceptual projects, for larger volume applications in the Middle East, are being designed with centrifugal compressors. Injection pressures can range from 34.5 bar (500 psi) to over 207 bar (3000 psi) depending upon the depth and permeability of the formation. Depleted reservoirs or deep aquifers are typically utilized. These “relatively” small projects can be designed and operated safely with existing technology. (Maddocks)

Figure 2.1 Location of CO2 Sources and Pipelines in the US

B.   CO2 Capture Technology

CO2 is typically captured from a process plant gas stream by contacting the stream with an appropriate solvent. The choice of solvent depends primarily on the pressure of that gas, its CO2 content, and the levels and types of contaminants contained in that gas. Low pressure (near atmospheric pressure) gas streams are typically treated with amine-based solvents that remove the CO2 by chemical reaction. High pressure gas streams (>3.6 bar (50 psi)) are typically treated with solvents that capture CO2 by physical absorption. Solvent regeneration to break the chemical bonds between the amine and CO2 is done by the use of heat, typically recovered from other plant process streams. CO2 is typically removed from the physical solvents by pressure reduction.

There are three relatively low capacity plants currently operating in the US that use monoethanolamine (MEA) solvent to capture CO2 for local uses including freezing chickens, carbonating soda pop, and manufacturing baking soda, at a cost of ~$140/ton CO2. The total amount of CO2 recovered in these plants is about 270 MT/day (300 tons/day). This is equivalent to the emissions from a very small (~15 MW) power plant.

Coal gasification plants that produce hydrogen, ammonia, and other fertilizers typically use physical solvents to remove CO2 and H2S from product gases. Most of these plants are located in China and South Africa. Some plants of this type operate in the US.

Oxyfuel is a combustion process under development at a number of locations.  It combusts fuel with oxygen which is diluted with captured and recycled CO2. There are several contaminants that must be controlled to specific levels including O2, N2, Ar, SO2, and H2O, to avoid problems with the CO2 capture system. (Schoff). The largest Oxyfuel development facility is a 50 MWt natural gas fired demonstration plant that is being planned for installation at the Kimberlina Power Plant near Bakersfield, CA. Other test facilities include a number of smaller coal-fired facilities including the B&W 30-MWt test facility in Ohio, a 30-MWt pilot plant under construction by Vattenfall, and several operating pilot-scale (~1 MWt) test units. (Schoff, Hustad)

Other technologies for CO2 capture are under development. Many pilot plant projects are planned and in development, including those that use chilled ammonia as a solvent. (Schoff)

One CCS demonstration now under way in the North Sea off the Norwegian coast is the Sleipner CO2 Injection Project. It is located on a drilling platform and utilizes an amine system to capture 1 million mt/y (1.1 million/tons/y) of CO2 that is then injected into a deep saline aquifer at 65 bar (840 psi).The objective of the project is to reduce the CO2 content of raw natural gas from 9 % to 2.5 %  to meet commercial sale specifications. The test program has been in operation since 1996 with a reliability level of 98-99%. (Miller)

The costs of CO2 capture from natural gas fired and coal fired power plants (IGCC plants and Oxyfuel plants) followed by pressurization to 150 bar (2200 psi) as reported at the Workshop by two authors are shown in Table 2.1.

Table 2-1 Cost of CO2 Capture

| Author | Hattenbach | Amick |
| --- | --- | --- |
|  | $/metric ton | $/metric ton |
| Natural Gas Combined Cycle | 83 |  |
| Supercritical Pulverized Coal | 67-68 | 40 |
| IGCC | 39 | 20 |
| Oxyfuel (new) | 48 |  |
| Oxyfuel (retrofit) | 67 |  |
| Coal to Liquids |  | 10 |
| Synthetic Natural Gas |  | 8 |

C.   CO2 Pipelines

As shown in Figure 2.1, existing networks of pipelines move CO2 from sources to markets. The purity of the CO2 used for EOR is >95 %. (Hattenbach) At this time, the major markets for CO2 are for Enhanced Oil Recovery (EOR) in the Permian Basin of Texas and New Mexico, the Gulf Coast, and the Weyburn fields in Saskatchewan, Canada. EOR operations in the Permian Basin utilize 0.043 bNm3/d (1.6 bcf/d) of CO2 to recover ~180,000 barrels per day (B/D) of incremental oil, which represents ~70 % of global CO2-EOR production. (Hustad)  In the U.S., a limited number of locations in Kansas, Mississippi, Wyoming, Oklahoma, Colorado, Utah, Montana, Alaska, and Pennsylvania also utilize CO2 injection to increase oil recovery. (Hattenbach, Kuuskraa).

The first CO2 pipeline in the US was constructed in 1974.  All of these pipelines utilize the same type of carbon steel pipe that is used for natural gas pipelines. These systems operate routinely without any significant or safety issues. Corrosion of carbon steel has been successfully avoided by maintaining the water content of the CO2 at very low levels to avoid formation of carbonic acid, which attacks carbon steel. (Kadnar)

- CO2 pipelines are protected from damage by the following procedures:

  - 24 hour monitoring by a Control Center

  - Membership in statewide one-call networks

  - Compliance with Common Ground Alliance Best Practices

  - Patrolled by air 26 times per year

- CO2 pipelines are protected from corrosion by:

  - Annual pipe to soil survey of pipeline

  - Five year cycle of Close Interval Surveys

  - Assessments of High Consequence Areas under Pipeline Integrity Management program (Kruuskaa)

Based on the assumed use of about 0.3 mt (0.33 tons) of CO2 /barrel of oil produced and production of about 250,000 B/D of oil by using CO2 injection (Kuuskraa), the total amount of CO2 carried by all the CO2 pipelines in the US is estimated at about 67,000 mt/day (75,000 tons/day). To put that number in perspective relative to the potential markets for CO2 capture for CCS purposes, a single 550 MW coal-fired power plant produces about 15,000 tons/day of CO2. (Schoff) Currently, US emissions of CO2 resulting from coal combustion amount to about 2100 MMT/y (2300 million tons per year) or about 5.7 million mt/day (6.3 million tons/day, equivalent to 400 coal-fired power plants, each with a capacity of 550 MW).

The costs of new CO2 pipelines have been estimated as follows:

100 miles of 24” pipe line with a capacity of (500 MMSCFD)

- Flat Dry Land					                                                $120,000,000

- Mountains					                                                $204,000,000

- High Populated Urban		  		                                                $250.000.000

- Offshore with a water depth of 46 m (150 ft.) – 61 m (200 ft)	         $1,680,000,000 (Kuuskraa)

IEA has proposed a combination of several approaches to stabilize the CO2 concentration in the atmosphere at 450 ppm by 2030. These include an annual reduction of CO2 emissions by 2.3 Gt/year by means of CCS. This would imply that the future amount of captured CO2 will be about the same as today’s natural gas production.

Twelve full-scale CCS projects are in the planning stage for Europe by 2012. These early projects will have individual pipelines. Interconnections among early projects are anticipated in 2015-2025. Looping of these pipelines is anticipated in 2025-2035 to create a CO2 pipeline ring similar to that now exists in Texas to serve the Permian Basin EOR market. (Bratfos)

D.  Delivered Cost of CO2

CO2 obtained from natural sources is now delivered commercially by pipeline to EOR sites at a price of about $1.25/MSCF ($24/metric ton, $22/ton). In comparison, the cost to compress and transport for 50 miles about 1.34 MM Nm3 (50 MMSCF/d) of CO2 recovered from high purity (>95%) man-made sources (natural gas processing plants, hydrogen production plants, etc.) will cost from $1.30 to $1.75/ MSCF or $25.50/mt ($23/ton) to $33.70/mt ($30/ton). The cost of compressing and transporting a similar amount of CO2 recovered from low purity (<15%) sources a similar distance would range from $2.85 to $4.00/MSCF or from $55.00/mt ($50/ton) to $77.00/mt ($70/ton). Of that total, the cost of capture is much higher than that of compression. Significant reductions are needed in both capture and compression cost for man-made sources of CO2 to compete with natural sources for EOR markets. (Hattenbach)

E.   Challenges of CO2 Transportation

The development of a national pipeline network equal in scope to the present natural gas pipeline network is a challenging task. An alternate approach is to focus on regional sequestration sites, and be proactive about siting issues so that new plants will be near sequestration sites. The use of CO2 for EOR is mature and the liability issues have been resolved. DOE cost goals for CO2 sequestration are very aggressive relative to currently estimated costs of capture and transportation. (Hattenbach)

For non-EOR sequestration to be commercially attractive, US industry needs visibility on:

- Value of emission reduction credit

- Regulations – Federal and State

  - Early action might be penalized

  - Economic - benefit or cost?

- Pore space ownership

- Liability issues

- Cost for capture and compression of man-made CO2 needs to be decreased (Hattenbach)

There are a number of concerns related to large scale CO2 transmission by pipeline:

- Root causes

  - Emergency blowdown of large dense phase inventories

  - Accidental denting

  - CO2 corrosion leaks in case of accidental intake of water

  - Material compatibility (elastomers, polymers)

  - Ductile fracture of pipeline (“un-zipping”)

- Consequences

  - Dispersion of concentrated CO2

  - Dispersion of toxic impurities

  - Pipeline damage/downtime

(Bratfos)

F. Properties of CO2 and Co-constituents Near the CO2 Critical Point

One of the conclusions reached by participants of the Workshop was that the use of currently available versions of the Equations of State (EOS) to predict the properties of supercritical CO2 which is contaminated with other compounds (i.e. A, N2, O2, CO, NH3, H2S,) at conditions near the critical point are not reliable enough for precise compression system designs. Several of the presentations commented on this issue as follows.

“GE has used the BWRS (Benedict-Webb-Rubin-Starling) EOS for the last 30 years: up to 300 bar on regular basis and up to 540 bar with CO2 + HC gas mixture in specific cases  .... also in the supercritical region. BWRS above 480 bar requires careful verification of literature data and is not suitable for liquid-vapor equilibrium calculations. Many existing CO2 EOS are optimized for pure CO2 but not for mixtures. To allow for regions not adequately covered by current EOS, GE is introducing a new thermodynamic model to improve predictability.” (Minotti)

“Better understanding of Phase behavior and confidence in EOS predictions” is needed.”(Maddocks)

“Equations of state near critical point… theories vary at high pressure also with co-constituents”.

(Miller)

“Compressibility is an issue at high pressure to stay away from liquid phase.”  (Kisor)

“Equation of state models for CO2 based mixtures have not been fully developed or validated. Large differences (19% variation) exist in gas properties predicted by standard equation of state models (API, RKS, HANS) and pure CO2 correlation models from 1000-2000 psia. EOS fall short on density and speed of sound especially with NIST supertrack program – is it applicable? “The needed actions are to perform more gas properties measurements of CO2 mixtures and refine equation of state near critical point and with mixtures.” (Moore)

”Equations of state are not good enough when we have water condensing out. Small amounts of impurities in CO2 change the location of the supercritical line. Better [pressure, volume, temperature] PVT data are needed on mixtures of CO2 and other gases.” (Hustad)

As a result of the deficiencies in the available data, larger margins than may be necessary are used by designers and manufacturers in their products. Better EOS have the potential to be used to lower equipment costs. As one illustration of the differences, Figure 2-2 (Moore) shows the variation in predicted density of CO2 obtained with various prediction methodologies.

Figure 2-2

G.   Compression Systems Machinery

1. Existing Compression System Machinery

Most of the large scale industrial experience with CO2 compression has been with CO2+H2S re-injection, fertilizer and hydrogen manufacturing, and CO2 pipelines. (Miller, Minotti, and Kisor) Reliability experience ranks centrifugal compressors highest, followed by integrally geared, and then reciprocating units. (Minotti)  GE has recently utilized supercritical compression (6 stages) to reach liquefaction conditions, followed by centrifugal pumping to enable pumping the supercritical fluid to the final required pressure. (Minotti)  Integrally geared machines achieve near-isothermal compression, which saves energy, but those machines have many more moving parts compared to reciprocating and centrifugal compressors. MAN Turbo compressors are used to pressurize CO2 at the Great Plains Coal Gasification plant in Beulah, ND for transmission by pipeline to the Weyburn oil fields in Saskatchewan, Canada a distance of more than 325 km. (200 miles).

CO2 compression requires a significant amount of energy to achieve a final pressure of 103 bara (1,500 psia) to 152 bara (2,200 psia) for pipeline transport or re-injection. For a typical 400 MW coal-fired plant, the typical CO2 flow rate is 120 mt/hr (132 tons/hr) to 140 mt/hr (154 tons/hr). The type of compressor selected is highly dependent on the starting pressure, which is approximately 1.3 bar (20 psia) to 34.5 bara (500 psia) for CO2 scrubbing of the fuel stream from an IGCC plant and approximately one bara (14.5 psia) from conventional pulverized coal power and Oxy-Fuel process power plants. Various types of compressors including ordinary and integrally geared centrifugal and reciprocating machines have been utilized to meet these compression service requirements depending on inlet and outlet pressures and volumetric flows. Reciprocating compressors are capable of achieving higher final pressures than centrifugal compressors, while centrifugal compressors can handle higher flow rates. For the large quantities of CO2 that must be handled in CCS applications, large capacity, single compression trains offer a significant cost advantage. (Moore)

Many vendors market the compressors that could be used in CO2 compression service for CCS projects. Dresser Rand, GE, and MAN Turbo, which are representative of vendors that produced very large compressors were invited to present information on their typical products. Participants in the Workshop included representatives of other compressor vendors and technology developers including ABB, Curtiss-Wright, Elliott, Florida Turbine Technologies, Mitsubishi Heavy Industries, Solar Turbines, Turblex, and others.

The compressor data presented by Dresser-Rand, GE and MAN Turbo is summarized in Table

2-2.

Table 2.2 – Representative Large Compressor Data

| Vendor | Dresser Rand | GE | MAN Turbo |
| --- | --- | --- | --- |
| Reference | Miller | Minotti | Kisor |
| Compressor type | Reciprocating, Centrifugal | Centrifugal | Integrally Geared Centrifugal |
| Centrifugal Compressors in service/ total power | 105/ ~300 MW total | 200+/up to 18 MW for largest unit |  |
| Maximum Discharge Pressure | Centrifugal 178 bar (2,580 psia) operating 309 bar (4,472 psia) to be delivered in late 2009 | 280 bara | 225 bar |
| Maximum inlet flow | 82,100 m3/hr (48,300 acfm) | 300,000 Nm3/hr (176,500 acfm) | 350,000 Nm3/hr (205,800 acfm) |
| Reciprocating Compressors in service/ total power demand | 227 units/ >395MW | 180+/ |  |
| Maximum Discharge Pressure | 426 bara (6,213 psia) | 750 bara |  |
| Maximum inlet flow | 7,300 m3/hr (4,300 acfm) | 19,000 Nm3/hr (11,300 acfm) |  |

Design issues for CO2 compressors include carbonic acid corrosion of carbon steel if water is present in the system. The use of stainless steel for any components in contact with wet CO2 eliminates the problem. Similarly, the presence of water containing CO creates iron carbonyl upon contact with carbon steel.  Again, the use of stainless steels solves the problem. Special O-ring materials are required to resist explosive decompression due to entrapped CO2 within the O-rings. (Miller)

Aerodynamic challenges include very high pressure ratio and compressibility and a wide range of flow coefficient stages. Additional challenges relative to rotor dynamics are the very high density of CO2 and destabilizing effects and predictability of compressor seal dynamic coefficients. (Minotti)

Integrally geared compressors can be optimized for each stage due to lower volume and higher pressure at each progressive stage. This attribute provides the ability to spin high pressure impellers at higher speed. It is possible to go to different speeds on each pinion and stage so that  very high (50,000) rpm are possible. The polytropic efficiency of these machines is in the high eighties. As a result of the potential to form liquid phases at high pressures, the final compression stages are not intercooled, so that the temperature is always maintained above the critical point to stay in gas regions. (Kisor)

A sketch of a recent design of a MAN Turbo integrally geared compressor is shown in Figure

2-3.

Figure 2.3 MAN Turbo Integrally Geared Compressor

2. R&D to Support Future Advancements in Compression Systems Machinery

Interstage Cooling/Liquefaction/Cryogenic Pumping

The high pressure ratios required in each turbine stage to ultimately reach the high total pressures required by CCS systems results in a significant amount of heat of compression. Compression systems must also be integrated with both the power production and CO2 capture plants to optimize heat integration. DOE-supported studies by SwRI, working with Dresser-Rand, have demonstrated that an isothermal compressor combined with cryogenic pumping offers the potential to significantly reduce compression power requirements by 20-35%. The goal of this R&D program is to develop an internally cooled compressor stage and qualify a liquid CO2 pump for CCS service

The focus of the internally cooled compressor stage program is to:

- Provide performance equivalent to an integrally geared compressor

- Achieve the high reliability of an in-line centrifugal compressor

- Reduce the overall footprint of the package

- Have less pressure drop than an external intercooler

The CO2 liquefaction process that SwRI has identified as being very promising in terms of reducing compression requirements significantly follows the steps listed below:

- Utilizes a refrigeration system to condense CO2 at about 17.2 bar (250 psia) and -20ºC (-36ºF).

- Liquid is then pumped from 17.2 bara (250 psia) to 153 bara (2,215 psi).

- Significantly less power is required to pump liquid compared to compressing a gas.

- The cost of the refrigeration system must be accounted for. (Moore)

GE is now using supercritical compression (4 stages) and centrifugal pumps and refrigeration at

-20 ºC (-36 ºF) to reduce power requirements by about 25 % in one specific application. (Minotti)

Advanced Compressors

Ramgen is developing an advanced compressor for CCS applications with the following:

- 100:1 CO2 compressor 2-casings/2-stages/intercooled

  - No aero Mach # limit

  - 10+:1 pressure ratio; 400°F temperature rise

  - 1400 fps tip speeds; Shrouded rotor design

- Single-stage, discrete-drive

  - Single stage per drive optimizes specific speed match

- “Compressor” heat exchanger cost can be eliminated

  - Eliminate or substantially reduce cooling tower requirement

  - Eliminate or substantially reduce cooling tower make-up water

  - 3x LMTD heat exchangers with 1/3 the surface area

The claimed attributes of this approach are:

    - 1/10th the physical size – facilitate space constrained retrofits

    - 1/2 the installation cost

    - Reduce CCS cost by 56 % from $64 to $28/tonne CO2 (Baldwin)

Dresser-Rand has recently begun supporting this program. (Miller)

H. Electric Drive Machinery

1. Existing Electric Drive Machinery

The oil and gas industry is following the world-wide trend to increased electrification with  a diverse range of applications for high power electric drives which require:

- High reliability/availability/maintainability

- High power

- High voltage

- High speed

- Ability to operate in harsh environments

(Zhang)

A variety of high megawatt direct electric drives are currently available for exploration, production, transport, and processing applications. However, further improvements in capabilities are needed to serve the market for remote sub-sea power located more than 100 miles off-shore in water with depths greater than 200 feet.

The relationship among speed and power rating for various segments of the electric drive market is shown in Figure 2.4.

Figure 2.4 Market Segments for Large Electric Drives (Zhang)

Among the requirements for this equipment are low ripple currents and low harmonics. GE is offering an integrated high speed motor/generator to the oil and gas markets with drive power needs of up to 15 MW. High speed, high power, direct drive systems eliminate the need for a gear box, which improves reliability.

Recent achievements reported by GE include:

- Replacement of LCI with ICGT drive systems reduces torque ripple by a factor of 3

- Move to high frequency integrated M/G operating at 11,000-17,000 rpm

- 35 MW output at 100 Hz with multi-thread parallel and interleaving control system design

High efficiency synchronous motors are an important approach to minimum total lifecycle costs for drive machinery, since the cost of the electricity used represents 74 % of total lifetime cost for these systems. 4-6 pole synchronous motors offered by ABB in the range of 10-60 MW feature high efficiency, low inrush current and variable power factor. (Kullinger)

Converteam offers Variable Motor Drive Systems in two power ranges, 2-32 MW and 10-100 MW. The lower power system, which uses MV- IGBT press pack technology, can be used with high speed motors, induction motors, and synchronous motors. The higher power system, which uses LCI – Thyristor technology, can be used with both synchronous motors and high speed synchronous motors. (Moran)

2. R&D to Support Future Advancements in Electric Drive Machinery

The market requirements for electric drive machinery are focused on the needs to operate at higher power ratings with even greater reliability and efficiency than today’s product offerings. The key to meeting these market demands lies in the realm of technology development that will allow commercial products to operate reliably at voltages above 10 kVA and frequencies above 10 kHz.

Drive component R&D needs include:

- Advanced stator and rotor cooling schemes

- Improved materials for high speed rotors, advanced design tools

- Advanced stator and rotor materials to handle corrosive gases

- Improved drive electronics

  - higher fundamental frequencies for high speed machines

  - improved controls and bandwidth to provide low torque ripple

- Tighter integration of compressor, motor and drive components and engineering

(Raju)

I. Drive Electronics and Components

1. Existing Drive Electronics and Components

Mechanical drives have been widely used in the past. They are available at high ratings and are independent of the requirements associated with electricity supply infrastructure. Compared to mechanical drives, electrical drives offer improved speed control, higher system efficiency, reduced maintenance, dynamic braking, the capability of short start-up time and load assumption, and elimination of the gear box that enables tight integration of drive motor with the compressor. Electrical drive challenges include the requirement of availability of on-site electricity and power ratings have to be met by both motor and frequency converter (“drive”). The integration advantages of electric drives include direct coupling of motor and compressor rotors thereby eliminating the gear box and the ability to cool motors with the flow of process gas. The power train can be levitated by magnetic bearings. As a result of these characteristics, there is the potential for substantial simplification of compression stations through the use of electric drives in place of mechanical drives.

Permanent magnet motor technology using rare-earth permanent magnet rotor poles, metallic retaining ring and magnetization after assembly, offer the benefits of robust manufacturing processes, no active rotor components, and minimal heating and thermal cycling. (Raju/Weeber)

The use of SiC based components in place of Si-based components can enhance the performance of semiconductor power devices by an order of magnitude for switching frequency and a factor of 5 for device voltage, as shown in Figure 2.5

Figure 2.5 Semiconductor Power Devices (Stevanovic)

Currently, there are no commercially available SiC devices that are capable of operating at 10 kV. Robust, reliable devices scaleable to >1 kA are also needed. The challenges facing currently available power modules include thermal limitations, electrical de-rating, and wirebond reliability. New soft magnetic materials have the advantages of minimizing hysteretic losses, minimizing eddy current losses and maximizing materials utilization. (Stevanovic)

Today’s commercial market for power conditioning devices, used primarily in Power Factor Correction (PFC) and solar power conversion applications, utilizes Si (silicon)-based 600-1200 V, 5 A-50 A components. Silicon Carbide (SiC) based components offer significant technical advantages relative to silicon components, which are summarized below:

- 10X Breakdown Field of Si

  - Tradeoff  higher breakdown voltage

  - Lower specific on-resistance

  - Faster switching

- 3X Thermal Conductivity of Si

  - Higher current densities

- 3X Bandgap of Si

  - Low ni     Low leakage current

  - Higher temperature operation

Today SiC based components are relatively expensive but larger production volumes and larger wafer sizes (4 inch diameter instead of 3 inch diameter) are resulting in continuous product cost reduction.

Recent field experience with SiC-based test components was reported at the Workshop by Cree. A 2.4 % increase in efficiency of a 3-phase solar inverter was achieved using Cree 1200 V SiC DMOSFETs in place of 1200 V Si IGBTs. Significant cost savings were achieved by reducing losses in power conversion efficiency. Switching losses with 3.3 kV SiC DMOSFET were more than >10X lower than with 3 kV Si IGBT at 125 °C. The 3.3 kV SiC DMOSFET is capable of 20 kHz switching operation. Early field data is showing a 10X lower failure rate than comparable silicon-based parts. (Palmour)

2. R&D to Support Future Drive Electronics and Components

Robust, reliable devices scaleable to >1 kA are needed. There are no commercially available 10 kV SiC devices. The challenges include:

- VON(T) for majority carrier devices

- Improving the yield of large MOS-gated (FET, IGBT) devices

- Gate oxide reliability, stability

- Bipolar degradation

There are no commercially available >10 kV, >1 kA modules. Design challenges include:

- Device interconnect for high currents and temperatures

- Materials CTE matching

- Fault tolerant to open/short failure

- High performance (top & bottom) device cooling

Development of new magnetic materials requires R&D to:

- Advance alloy theory and modeling to impact: saturation magnetization, anisotropy magnetostriction

- Apply advanced magnetic and structural probes to magnetic materials

- Develop new process routes to achieve desired microstructures

- Validate material performance in pilot-scale processing (Stevanovic)

To provide the needed capabilities for 10 kV devices, SiC IGBTs, GTOs and PiN Diodes are needed.  This will require:

- SiC production and reliability proven at low voltages (600-1200 V) and running in high volume

- SiC MOSFETs nearing production at 1.2 kV, and 3.2 kV – 10 kV devices are proven and circuit demos show incredible performance

- For higher voltage (>10 kV), GTOs and IGBTs have been demonstrated

- SiC will enable high voltage drive trains with efficiencies and frequencies far in excess of what can be achieved in Si  (Palmour)

3. Prioritization of Potential R&D Projects

Workshop participants were asked to suggest research projects for consideration by the group and subsequent prioritization. Similar suggestions were combined with one another to reduce the number of proposed projects. A total of 33 projects were suggested which were organized into seven categories.

The voting process allocated 100 total votes to each participant. Individuals could distribute their votes among as many projects as they wished, but were not allowed to award more than thirty votes to any one project. As a result of time constraints, participants were asked to submit their completed ballots by email. A total of 37 individuals participated. Employees of the sponsoring organizations (DOE, NIST, and EPRI) did not participate in the prioritization process.

Tables 3.1 presents the distribution of total votes among the seven categories. Table 3.2 lists the ten highest ranked projects. Tables 3.3 through 3.9 present the total votes for R&D projects in each of the seven categories.

The highest ranked category and highest ranked projects related to the need to have more accurate prediction methodologies available for calculating the thermodynamic properties of mixtures of CO2 containing relatively small concentrations of contaminants totaling less than about 5 %. This category and topic were followed in priority by projects to improve integration of the capture and compression systems.

Table 3.1 Category Rank Order

| Category Rank Order | Total Votes |
| --- | --- |
| 1. Properties of CO2 and Co-constituents | 914 |
| 2. Integration of CO2 Capture and Compression | 726 |
| 3. Compression Systems Machinery and Components | 690 |
| 4. Electric Drive Machinery | 545 |
| 5. Pipeline Issues | 456 |
| 6. Drive Electronics and Components | 326 |
| 7. Impacts of Legislation on CCS | 43 |

Table 3.2 R&D Project Rank Order

| R&D Project | Total Votes |
| --- | --- |
| 1. Perform more gas properties measurements of CO2 mixtures | 435 |
| 2. Improve Equations of State | 401 |
| 3. Optimize integration of a CO2 capture/compression systems together with the power plant | 280 |
| 4. Comparison and evaluation of compression-liquefaction and pumping options and configurations | 204 |
| 5. Higher voltage, higher power, and speed machines and drives. | 165 |
| 6. Install test coupons in existing CO2 pipelines to obtain corrosion data, then develop CO2 product specifications | 150 |
| 7. Determine optimal machine types, speeds, needed voltages, etc. for CO2 compressors | 143 |
| 8. Establish allowable levels of contaminants in CO2 pipeline and/or compressors | 120 |
| 9. Compressor heat exchanger data for power plant applications including supercritical fluids | 117 |
| 10. Integrate utilization of waste heat to improve cycle efficiency | 113 |

Table 3.3 Voting Distribution - Properties of CO2 and Co-constituents

| Category | Total R&D Project Votes (Rank Order) | R&D Project Descriptions |
| --- | --- | --- |
| 1. Properties of CO2 and Co-constituents Total Category Votes = 914 | 435 (1) | Perform more gas properties measurements of CO2 mixtures - Collect experimental PVT and VLE data and develop correlations for systems with 60-100 % CO2, 0-40 % H2S, 0-5 % Ar, and 0-5 % N2, H2O - Develop an understanding of the impact of Ar and N2 and the pressure required to obtain dense phase supercritical CO2 - Thermodynamic properties of CO2 and ranges of impurities expected in CCS applications within vapor dome is liquid (also supercritical) - Variable speed of sound pulsation models (real gas effects) - Provide experimental data of CO2 and co-constituents properties including (NH3)2 at pressures ranging from 5-2500 psia and then develop simulation model with experimental data |
|  | 401 (2) | Improve Equations of State - Equation of State predictions at all pressures with water present at various concentrations - Establish standard equations of state usage in analysis - Refine equation of state near critical point and with mixtures from 1 psia up to 11,000 psia |
|  | 78 (21) | Define compositions/pressures for power plants, reinjection recycle, pipeline |

Table 3.4 Voting Distribution - Integration of CO2 Capture and Compression

| Category | Total R&D Project Votes (Rank Order) | R&D Project Descriptions |
| --- | --- | --- |
| Integration of CO2 Capture and Compression | 280 (3) | Optimized integration of CO2 capture/compression systems together with the power plant |
| Total Category Votes = 726 | 161 (6) | Evaluate cost/benefits for various CO2 capture options based on various CO2 impurity specs (10 ppm, 50 ppm, 100 ppm, 1000 ppm) |
|  | 113 (11) | Integrate utilization of waste heat to improve cycle efficiency |
|  | 91 (16) | Evaluate alternate CO2 compressor drives (steam and gas turbines) |
|  | 81 (20) | IGCC Demonstration project with CO2 capture to reduce risk and enhance workability |

Table 3.5 Voting Distribution - Compression Systems Machinery and Components

| Category | Total R&D Project Votes (Rank Order) | R&D Project Descriptions |
| --- | --- | --- |
| Compression Systems Machinery and Components | 204 (4) | Comparison and evaluation of compression-liquefaction and pumping options and configurations |
| Total Category Votes = 690 | 117 (10) | Compressor heat exchanger data for power plant applications including supercritical fluids |
|  | 99 (15) | Advanced rotating equipment clearance control and sealing technology demonstration |
|  | 91 (16) | Axial compression system demonstrator for 13 k ton/day |
|  | 90 (18) | Design very large axial compressors to provide initial stages of compression followed by conventional HP compressors |
|  | 48 (25) | Integrated back-pressure steam turbine and CO2 compressor |
|  | 30 (28) | Document duty cycle requirements for reference plant |
|  | 11 (31) | Improve reliability of recipe EOR recycle compressors, i.e. valve reliability, lubrication |

Table 3.6 Voting Distribution =- Electric Drive Machinery

| Category | Total R&D Project Votes (Rank Order) | R&D Project Descriptions |
| --- | --- | --- |
| Electric Drive Machinery | 165 (5) | Higher voltage, higher power, and speed machines and drives. |
| Total Category Votes = 545 | 143 (8) | Determine optimal machine types, speeds, needed voltages, etc. for CO2 compressors |
|  | 111 (12) | Tighter integration of compressor, motor and drive components and engineering. |
|  | 56 (23) | Improve drive electronics - higher fundamental frequencies for high speed machines, improved controls, and bandwidth to provide low torque ripple |
|  | 45 (26) | Advanced Stator and Rotor cooling schemes |
|  | 15 (28) | Improve materials for high speed rotors and advanced design tools |
|  | 10 (32) | Advanced Stator and Rotor materials to handle corrosive gases |

Table 3.7 Voting Distribution - Pipeline Issues

| Category | Total R&D Project Votes (Rank Order) | R&D Project Descriptions |
| --- | --- | --- |
| Pipeline Issues | 150 (7) | Install test coupons in existing CO2 pipelines to obtain corrosion data, then develop CO2 product specifications including H2O, O2, NH3, TEG, Amines |
| Total Category Vote - 456 | 120 (9) | Establish allowable levels of contaminants in CO2 pipeline and/or compressors |
|  | 111 (12) | Perform optimization of pipeline booster stations. Station spacing, liquid vs. gas, driver selection |
|  | 75 (22) | Perform further corrosion studies on the effects of moisture on pipeline corrosion |

Table 3.8 Voting Distribution - Drive Electronics and Components

| Category | Total R&D Project Votes (Rank Order) | R&D Project Descriptions |
| --- | --- | --- |
| Drive Electronics and Components Total Category Votes= 326 | 108 (14) | Development of SiC components and inverter modules for cost effective variable speed drive and cost effective electrically driven compressors - Manufacturing and cost reduction for SiC power modules - Determine and develop optimal device type for CO2 compression application |
|  | 88 (19) | Integration of CO2 compression electric drive with power plant electrical system |
|  | 55 (24) | Development and demonstration of high voltage, high frequency motor drives |
|  | 45 (26) | Integration of pipeline pumping station motor drive with electrical grid |
|  | 25 (29) | High frequency transformer magnetic materials: nano-crystalline magnetic materials |
|  | 5 (33) | High voltage, high current module packaging - Better thermal performance - Better reliability |

Table 3.9 Voting Distribution - Effects of Legislation on CCS

| Category | Total R&D Project Votes (Rank Order) | R&D Project Descriptions |
| --- | --- | --- |
| Effects of legislation on CCS Total Category Votes = 43 | 43 (27) | Determine practical effects of new legislation on CCS (after new legislation is in place) |

4. List of Workshop Presentations

Phil Amick, ConocoPhillips; Gasification Project Outlook

Peter Baldwin, RamGen; Ramgen Power Systems

Hans Axel Bratfos, DNV; Risk Aspects Related to Pipeline Transmission of CO2

Ray Hattenbach, Blue Source LLC; Future Market Drivers for CO2 Compression Equipment

Carl Hustad, CO2 Global; CO2 Compression for Advanced Oxy-Fuel Cycles

Joy Kadnar, US Department of Transportation; CO2 Transportation Via Pipelines

Kevin Kisor, MAN Turbo; Centrifugal Compressors for High Pressure CO2 Applications

Dan Kubek, Gas Processing Solutions; Large CO2 Sources and Capture Systems

Kenneth Kullinger, ABB; High-megawatt Electric Drive Motors

Vello Kuuskraa, Advanced Resources International; Summary of Results from the EPRI Workshop on Costs of CO2 Storage and Transportation

Jim Maddocks, Gas Liquids Engineering; Gas Processing

Harry Miller, Dresser Rand; Carbon Dioxide Compression

Marco Minotti, GE; CO2 Compression Capabilities

Jeff Moore, SwRI; Research and Development Needs for Advanced Compression of Large Volumes of Carbon Dioxide

Steve Moran, Converteam; Multi-megawatt Motor Drive Technology Electronics

John Palmour, Cree; Future High-Voltage Silicon Carbide Power Devices

Ravi Raju (for Konrad Weeber), GE Research; Advanced Electric Machines Technology

Ron Schoff, EPRI; Introduction of Large Power Plants with CO2 Capture and Compression

Ljubisa Stevanovic, GE Energy; Advanced Electronic Components for High Speed, High-megawatt Drives

Richard Zhang, GE Oil and Gas; High-megawatt Electric Drive Applications in Oil and Gas

5. Appendices

5a. Workshop Agenda

Workshop on Future Large CO2 Compression Systems

Sponsored by

DOE Office of Clean Energy Systems, EPRI, and NIST

Dates

March 30-31, 2009

March 30, 2009

- Future Market Outlook for CO2 Compression and Sequestration

- Existing Industry Experience with CO2 Compression

- Approaches to Improve Cost, Efficiency, Availability, and Safety

March 31, 2009

- Advanced Compressor Machinery R&D Needs

- Advanced Electric Drive Technology R&D Needs

- Identify and Prioritize R&D Needed for Future CO2 Compressors

| Time | Topics |
| --- | --- |
|  | First Day (March 30) |
| 8 AM | Registration and Breakfast |
| 8:30 AM | 1.0 Opening Welcome - Introduction of Participants, Opening Remarks Al Hefner, NIST; Pete Rozelle, DOE; Rob Steele, EPRI - Review of Workshop Objectives - Ron Wolk 1.2 Keynote Speakers - Future Market Drivers for CO2 Compression Equipment; Ray Hattenbach, Blue Source LLC - Introduction of Large Power Plants with CO2 Capture and Compression; Ron Schoff, EPRI |
| 10:00 AM | Break |
| 10:20 AM | 2.0 Oil and Gas Industry Experience with CO2 Compressors and Pipelines - Joy Kadnar, US Department of Transportation; CO2 Transportation Via Pipelines - Hans Axel Bratfos, DNV; Risk Aspects Related to Pipeline Transmission of CO2 - Dan Kubek, Gas Processing Solutions; Large CO2 Sources and Capture Systems - Vello Kuuskraa, Advanced Resources International; Summary of Results from the EPRI Workshop on Costs of CO2 Storage and Transportation 2.1 Panel Discussion - Jim Maddocks, Gas Liquids Engineering - Phil Amick, ConocoPhillips |
| 12:15 PM | Lunch |
| 1:15 PM | 3.0 Compressor Vendor Perspective on Changes in Compression Cycle, Machinery, and CO2 Capture System to Increase Energy Efficiency - Harry Miller, Dresser Rand; Dresser-Rand Centrifugal and Reciprocating Compressor Technology and Experience with CO2 Compression Applications. - Kevin Kisor, MAN Turbo; Compressors for High Pressure CO2 Applications - Marco Minotti, GE; CO2 Compression Capabilities |
| 3 PM | Break |
| 3:30 PM | 4.0 Electric Drive Compressor Potential for Improvement in Capitol Cost, Power Requirements, Availability, and Safety - Richard Zhang, GE Oil and Gas; High-megawatt Electric Drive Applications in Oil and Gas - Kenneth Kullinger, ABB; High-megawatt Electric Drive Motors - Steve Moran, Converteam; High-megawatt Motor Drive Electronics |
| 5 PM | Adjourn |
| 6:30 PM | EPRI-Hosted Workshop Dinner |
|  | Second Day (March 31) |
| 8 AM | Breakfast |
| 8:30 AM | 5.0 Review Workshop Charge to Identify and Prioritize R&D for Future CO2 Compression Systems - Ron Wolk |
| 8:40 AM | 6.0 Advanced Compressor Machinery Future R&D Needs - Jeff Moore, SwRI; Research and Development Needs for Advanced Compression of Large Volumes of Carbon Dioxide - Carl Hustad, CO2 Global; CO2 Compression for Advanced Oxy-Fuel Cycles - Peter Baldwin, RamGen; Ramgen Overview and Status Update |
| 10 AM | Break |
| 10:30 AM | 7.0 Advanced Electric Drive Compressor Future R&D Needs - Ravi Raju for Konrad Weeber, GE Research; Advanced PM and Synchronous Machine Technology - Ljubisa Stevanovic, GE Energy; Advanced Electronic Components for High Speed, High-megawatt Drives - John Palmour, Cree; Future High-Voltage SiC Power Device Manufacturing Technology |
| Noon | Lunch |
| 1 PM | 8.0 Compilation of Potential R&D Areas Workshop Participants, (Ron Wolk, Facilitator) - Capture and Compression System Modifications - Potential Compressor Machinery Improvements - Potential Electric Drive Compressor Developments - Potential Improvements in High Power Electronics |
| 2:00 PM | R&D Prioritization Exercise Workshop Participants, (Ron Wolk, Facilitator) |
| 3:00 PM | Adjourn |

5b. Workshop Participants

| Last Name | First Name | Company | E-mail |
| --- | --- | --- | --- |
| Agarwal | Anant | Cree Inc. | Anant_Agarwal@cree.com |
| Alsup | Charles (Chuck) | DOE/NETL | calsup@netl.doe.gov |
| Altpeter | Paul D. | Air Products and Chemicals | altpetpd@airproducts.com |
| Ames | Robin | NETL - DOE | Robin.Ames@NETL.DOE.GOV |
| Amick | Phil | Conoco-Phillips | Phil.Amick@ConocoPhillips.com |
| Anderson | Malcolm | Southern California Edison | malcolm.anderson@sce.com |
| Baldwin | Peter | Ramgen Power Systems | pete_baldwin@ramgen.com |
| Beermann-Curtin | Sharon | DARPA | Sharon.Beermann-curtin@darpa.mil |
| Behnke | Paul W. | Bechtel | PWBEHNKE@BECHTEL.COM |
| Bender | William | Technology & Management Services | wbender@tms-hq.com |
| Biondo | Sam | DOE Headquarters | SAMUEL.BIONDO@HQ.DOE.GOV |
| Boal | Charles | Elliott Company | cboal@elliott-turbo.com |
| Bower | Richard D. (Rick) | Alter NRG Corp. | rbower@alternrg.ca |
| Bratfos | Hans Axel | Det Norske Veritas (DNV) Energy | hans.axel.bratfos@dnv.com |
| Brent | Richard | Solar Turbines | cochraham_nicole@solarturbines.com |
| Brostmeyer | Joseph | Florida Turbine Technologies | jbrostmeyer@fttinc.com |
| Brown | Barry | Florida Turbine Technologies | bbrown@fttinc.com |
| Bygrave | Jonathan W. | Rolls-Royce Energy Systems, Inc. | jonathan.bygrave@rolls-royce.com |
| Colangelo | Mike | A.O. Smith | Mike.Colangelo@AOSEPC.COM |
| Davis | Gary | Elliott Company | gdavis@elliott-turbo.com |
| Dennis | Richard | US DOE - NETL | Richard.dennis@netl.doe.gov |
| Ericsen | Terry | ONR | ericset@onr.navy.mil |
| Faller | Wolfgang | Solar Turbines, Inc. | Faller_Wolfgang@solarturbines.com |
| Feier | Ioan | Battelle Memorial Institute | feieri@battelle.org |
| Gaspar | Jeff | Converteam | jeffrey.gaspar@converteam.com |
| Hannon | Cesar | ALSTOM Power - Environmental Control Systems | cesar.hannon@power.alstom.com |
| Hattenbach | Ray P | Blue Source LLC | rhattenbach@bluesource.com |
| Hefner | Allen | NIST | allen.hefner@nist.gov |
| Hoffman | James | DOE/NETL | jhoffman@netl.doe.gov |
| Hood | Colleen | NIST | colleen.hood@nist.gov |
| Hoover | Ron | Southern California Edison | Ronald.hoover@sce.com |
| Hopkinson | David | DOE | david.hopkinson@hq.doe.gov |
| Horiba | Junichi | Mitsubishi Heavy Industries America, Inc. | junichi_horiba@mhiahq.com |
| Huntington | Richard | ExxonMobil Research & Engineering Co. | Richard.huntington@exxonmobil.com |
| Hustad | Carl-W. | CO2-Global | cwh@co2-global.com |
| Jones | Russell | Florida Turbine Technologies | rjones@fttinc.com |
| Kadnar | Joy O. | U.S Department of Transportation | Joy.kadnar@dot.gov |
| Kerth | Jason | Dresser-Rand | Jkerth@dresser-rand.com |
| Kindt | Jack T. | Air Products and Chemicals | kindtjt@airproducts.com |
| Kisor | Kevin W. | MAN Turbo Inc. USA | Kevin.Kisor@manturbo-us.com |
| Kubek | Dan | Gas Processing Solutions LLC | dan.kubek@sbcglobal.net |
| Kullinger | Kenneth | ABB | Kenneth.kullinger@se.abb.com |
| Kuuskraa | Vello A. | Advanced Resources International, Inc. | vkuuskraa@adv-res.com |
| Lai | Jason | Virginia Tech | laijs@vt.edu |
| Le | Patrick H. | US DOE/NETL | ple@netl.doe.gov |
| Ludwiczak | Christian | E.ON Ruhrgas AG | Christian.ludwiczak@eon-ruhrgas.com |
| Maddocks | Jim | Gas Liquids Engineering Ltd. | jmaddocks@gasliquids.com |
| Miller | Harry F. | Dresser-Rand | hmiller@dresser-rand.com |
| Minotti | Marco | GE Oil & Gas | Marco.minotti@ge.com |
| Moore | Jeff | Southwest Research Institute | jeff.moore@swri.org |
| Moran | Steven | Converteam Naval Systems | steven.moran@converteam-navsys.com |
| Muraskin | David | Alstom Power | David.j,Muraskin@power.alstom.com |
| Olliver | Richard A. | CH2M Hill | rolliver@ch2m.com |
| Omatick | Todd | Elliott Company | tomatick@elliott-turbo.com |
| Palmour | John | Cree, Inc. | john_palmour@cree.com |
| Peralta-Solorio | David | E.ON Engineering Ltd. | David/peralta-solorio@eon-engineeringuk.com |
| Pollard | Alicia | Elliott Company | apollard@elliott-turbo.com |
| Quedenfeld | Heather | U. S. DOE | heather.quedenfeld@netl.doe.gov |
| Raines | Tom | Alstom Power Environmental Control Systems | thomas.s.raines@power.alstom.com |
| Raju | Ravisekhar(Ravi) | General Electric Company | raju@crd.ge.com |
| Royal | John | Praxair Inc. | John_royal@praxair.com |
| Rozelle | Peter | DOE Headquarters | peter.rozelle@hq.doe.gov |
| Sadok | Richard D. | ConocoPhillips | Richard.D.Sadok@Conocophillips.com |
| Schoff | Ronald L. | Electric Power Research Institute | rschoff@epri.com |
| Shaffer | Ron | Curtiss-Wright EMD | Rshaffer2@curtisswright.com |
| Shelton | Walter W. | U.S. DOE / NETL/ OSAP | Walter.Shelton@NETL.DOE.GOV |
| Soghomonian | Dr. Zareh | BMT Syntek Technologies | zsoghomonian@bmtsyntek.com |
| Steele | Rob | EPRI | rsteele@epri.com |
| Stevanovic | Ljubisa | GE Global Research Center | stevanov@crd.ge.com |
| Temofonte | Peter | Turblex, a Siemens Company | peter.temofonte@siemens.com |
| Tipsword | Robert | Core Energy, LLC | btipsword@coreenergyllc.com |
| Wadas | Brian | G E Oil & Gas | brian.wadas@ge.com |
| Wen | Harvey | Bechtel Power Corp | hwen@bechtel.com |
| Wheeler | Emily | DOE | Emily.wheeler@hq.doe.gov |
| Williams | Ben | Ariel Corporation | bwilliams@arielcorp.com |
| Wolk | Ronald H. | Wolk Integrated Technical Services | ronwolk@aol.com |
| Worst | Jeff | Curtiss-Wright | jworst@curtisswright.com |
| Wung | Dr. Peter | A. O. Smith Electrical Products Company | Peter.wung@aosepc.com |
| Zgrabik | Larry | ABB Inc. | larry.l.zgrabik@us.abb.com |
| Zhang | Richard | GE Oil & Gas | zhangr@ge.com |