Grasp Cycle Time

Metric

Grasp cycle time is a measure of the minimum time required for a robotic hand to achieve full closure from a known pre-grasp configuration and to return to the pre-grasp configuration from the grasp position. This measure will yield information regarding a particular hand’s closing/opening speed capabilities.

Dependencies

Closing/Opening time is a function of the hand’s actuator capabilities, motion controllers, mechanical design and grasp configuration.

Test Method

Artifact:  NIST Split Cylinder Artifact or application relevant artifact with internal force sensing.

Description:

Of the previously listed dependencies, only the grasp configuration and object size are assumed controllable. Two common grasp types can be chosen for investigation – pinch and wrap. The pinch grasp allows for measuring closing/opening performance associated with precision grasping while the wrap grasp allows for measuring performance associated with power grasping. The grasp cycle of a wrap grasp is depicted in Figure 1.  Artifact sizing should be chosen based on the intended application and the parts being handled. Otherwise, a reference artifact can be used to facilitate benchmarking across a variety of robotic hands.

Start Release 1			        Fully Open			Start Release 2

Figure 1:  Split cylinder artifact in zero degree orientation relative to power grasp stages

Performance Measures:

For each set of instantaneous force readings F1, F2, … Fi from the NIST cylindrical artifact or a similar artifact design, determine the total time between quasi-static closure force attained to yield a grasp cycle time Tgrasp_cycle,

Tgrasp_cycle = Tstop-Tstart

Tstart and Tstop are chosen as the first indication of grasp release for two subsequent grasp cycles performed (see Figure 2). Quasi-static grasp forces are chosen for evaluation as they remove impact effects and give a more accurate estimate of the time required to attain an object.  For thorough experimentation, several runs should be conducted to compute Tgrasp_cycle using two quasi-static grasp force events over two grasp cycles.   Then compute the grasp cycle time mean and 95% confidence intervals.

Figure 2: Depiction of dynamic and quasi-static force regions during grasp cycles.

Example Implementation (use artifact and touch sensors)

Test Setup:

Robot 2 is used with the 50.8 mm (2 inch PVC pipe) split cylinder artifact   Force data was captured from two load cells while fully opening and closing the robotic hand in a power grasp around the artifact 32 times for the 0 degree and 90 degree orientations.  The test setup is shown in Figure 3.

Figure 3:  50.4 mm (2 inch) diameter split cylinder artifact oriented at 0 degrees with Robotic Hand 2 performing wrap grasp cycles.

Results:

A data plot of F1 and F2 from the split cylinder artifact load cells throughout the 32 grasp cycles, is shown in Figure 4.  The mean quasi-static grasp forces were extracted for each data set.   Next, the mean and 95% confidence intervals for the force data collected in the zero degree orientation and the L2 norms are computed for both hands. The results are shown for Hand 2 in Table 1.

Figure 4:  Shows the load cell forces for the 50.4 mm (2 inch) diameter split cylinder artifact oriented at 0 degrees created by grasp cycles from Robotic Hand 2

Table 1. The mean and 95% confidence intervals of the internal grasp force for Hand 2.

|  | Tgrasp_cycle |  |
| --- | --- | --- |
| Robot | Mean (s) | 95% Confidence Interval (s) |
| Robot 2 | 3.8207 | [3.6432,3.9898] |

Data:

| Data File Archive: | Grasp Cycle Time.zip |
| --- | --- |
| Data Files: | Hand2_Grasp_Cycle_Time.csv |
| File Format: | ASCII, comma delimited |
| Data Values: | F1 and F2 (one set per line) |
| Units: | Newtons |
| Data Sample Rate: | 1 kHz |